Lab 5 - Introduction to Modeling and Verification of Digital Systems

ZLATEVSKI Andrej - REVERTER Jordi

Group 11

April 2026

NOTE: You can see the descriptions of the Controller, Counter and System for the FPGA synthesis in the Appendix.

# 1. Updated descriptions

# In the System description, we transformed the top-level entity into an FPGA-ready interface by adding the board I/O ports: LEDR(5 downto 0), LEDG(2 downto 0), and seg_val_hex0/seg_val_hex1/seg_val_hex2. We also added the chip_pin attributes for reset, clock, switches, LEDs, and 7-segment displays so Quartus can bind signals to physical DE1 pins. Another key addition is the clock divider process that derives ck_1Hz from the 50 MHz board clock; this slower clock is now used to drive the Robot and both counters for observable behavior on hardware. Finally, we updated component declarations and port maps so the controller and counters now export their display/LED signals to the top level.

In the Robot (controller) description, we extended the entity with FPGA output ports: LEDR, LEDG, and seg_val_hex0. We modified reset handling to active-low reset (reset = '0'), as required by the lab for the push-button behavior. For visualization on the board, we added direct input-to-LED mapping on red LEDs: LEDR(5..0) <= athome, findfood, lostfood, closetofood, success, scantimeup. We also added output-to-LED mapping on green LEDs: LEDG(2..0) <= rest, search, food. To display FSM progress, we added a state decoder process assigning 7-segment codes on seg_val_hex0 for states 0 to 8 (idle to deposit), using the encoding table requested in the lab sheet.

In the Count (counter) description, we extended the entity with seg_val_hex output for 7-segment display. We changed the internal counter type to natural range 0 to 15 for FPGA display compatibility and updated reset logic to active-low reset. We also added a counter-to-7-segment decoding process so each counter value is visible on HEX displays through seg_val_hex (including the lab’s decimal encodings, with continuation for A–F values).

Overall, the original behavioral logic was preserved, and the main FPGA synthesis additions are the physical pin mapping, human-visible outputs (LEDs + 7-segment displays), 1 Hz clock generation, and active-low reset adaptation.

# 2. Synthesis Results

Before the synthesis results, we recall what a CLB is. A CLB (Configurable Logic Block) is a small programmable unit in a chip used to perform logic operations. Each CLB contains 1 LUT to compute logic functions and 1 flip-flop to store one bit of data.

Results from the compilation of the top level entity (System.vhd)

In the image we see that the number of CLBs available is 18752 and used ones are 92. The number of LUTs is 18752 and used ones are 89. The number of flip-flops is 18752 and used ones are 44.

This latter number comes from the 26 bits used for encoding the state of the process to slow down the clock due to VARIABLE cnt : INTEGER RANGE 0 TO 67108863 which is log2(67108863) = 25.99 ~ 26. Then, for each counter we use 5 bits, 4 for the counter variable and 1 for the state. And for encoding the state of the FSM, the one-hot encoding is used in FPGA synthesis. With this encoding we expect to have 9 bits, but 8 bits are used because the state idle is encoded as a sequence of 0s. So, that's why we get 44 instead of 45 (the expected value). Quartus uses this as an optimization, removing 1 “redundant” register. Note that each bit corresponds to one flip-flop.

For the ASIC synthesis, 9 bits were used for one-hot encoding and taking into account that we didn’t have the process to slow the counter, the number is similar. For the ASIC we get 19 (assuming one-hot encoding), and for the FPGA without taking into account the clock slow down process, we get 18.

# 3. Appendix

Here you can find the VHDL files of the controller, counter and system:

CONTROLLER

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.numeric_std.all;

entity Robot is

port(reset, clk,

findfood, -- location of the food has been identified

closetofood, -- moving towards the food

lostfood, -- location of the food has been lost

scantimeup, -- scan time has expired

success, -- the food has been grabbed, or deposited

athome, -- control is given to exit the arena

aboverestth, abovesearchth : in std_logic;

rest, search, food : out std_logic;

LEDR : out std_logic_vector(5 downto 0);

LEDG : out std_logic_vector(2 downto 0);

seg_val_hex0: out INTEGER range 0 to 127);

end Robot;

architecture intf of Robot is

type States is (idle, resting, randomwalk, homing, movetofood, scanarena, grabfood, movetohome, deposit);

signal state, nextstate: States;

begin

-- expose the 6 inputs on red LEDs

LEDR(5) <= athome;

LEDR(4) <= findfood;

LEDR(3) <= lostfood;

LEDR(2) <= closetofood;

LEDR(1) <= success;

LEDR(0) <= scantimeup;

-- next state

process(state, findfood, closetofood, lostfood, scantimeup, success, athome, aboverestth, abovesearchth)

begin

case state is

when idle => nextstate <= resting;

when resting =>

if aboverestth = '1' then nextstate <= randomwalk;

else nextstate <= resting;

end if;

when randomwalk =>

if (abovesearchth = '1') then nextstate <= homing;

elsif (findfood = '1') then nextstate <= movetofood;

else nextstate <= randomwalk;

end if;

when movetofood =>

if (abovesearchth = '1') then nextstate <= homing;

elsif (closetofood = '1')  then nextstate <= grabfood;

elsif (lostfood = '1')  then nextstate <= scanarena;

else nextstate <= movetofood;

end if;

when scanarena =>

if (abovesearchth = '1') then nextstate <= homing;

elsif (findfood = '1') then nextstate <= movetofood;

elsif (scantimeup = '1') then nextstate <= randomwalk;

else nextstate <= scanarena;

end if;

when homing =>

if athome = '1' then nextstate <= resting;

else nextstate <= homing;

end if;

when grabfood =>

if success = '1' then nextstate <= movetohome;

else nextstate <= grabfood;

end if;

when movetohome =>

if athome = '1' then nextstate <= deposit;

else nextstate <= movetohome;

end if;

when deposit =>

if success = '1' then nextstate <= resting;

else nextstate <= deposit;

end if;

end case;

end process;

-- updating of the state register:

process(reset, clk)

begin

-- asynchronous reset (active low):

if (reset = '0') then state <= idle;

-- otherwise, on clock rising edges:

elsif (clk'event and clk='1') then

state <= nextstate;

end if;

end process;

-- output function for rest

process(state, success, athome)

begin

if (state=idle) then

rest <= '1';

LEDG(2) <= '1';

elsif (state=homing) and (athome='1') then

rest <= '1';

LEDG(2) <= '1';

elsif (state=deposit) and (success='1') then

rest <= '1';

LEDG(2) <= '1';

else

rest <= '0';

LEDG(2) <= '0';

end if;

end process;

-- output function for food

process(state, closetofood, abovesearchth, lostfood)

begin

if (state=movetofood) and (abovesearchth='0') and (lostfood = '0') and (closetofood='1') then

food <= '1';

LEDG(0) <= '1';

else

food <= '0';

LEDG(0) <= '0';

end if;

end process;

-- output function for search

process(state, aboverestth)

begin

if (state=resting) and (aboverestth='1') then

search <= '1';

LEDG(1) <= '1';

else

search <= '0';

LEDG(1) <= '0';

end if;

end process;

-- display current FSM state on HEX0

process(state)

begin

case state is

when idle => seg_val_hex0 <= 2#0000001#;  	-- 0

when resting => seg_val_hex0 <= 2#1001111#;   -- 1

when randomwalk => seg_val_hex0 <= 2#0010010#;-- 2

when homing => seg_val_hex0 <= 2#0000110#;-- 3

when movetofood => seg_val_hex0 <= 2#1001100#; -- 4

when scanarena => seg_val_hex0 <= 2#0100100#;	-- 5

when grabfood => seg_val_hex0 <= 2#0100000#;  -- 6

when movetohome => seg_val_hex0 <= 2#0001111#;-- 7

when deposit => seg_val_hex0 <= 2#0000000#;   -- 8

end case;

end process;

end intf;



COUNTER

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.numeric_std.all;

-- Counter entity to manage the thresholds

entity Count is

generic(threshold: natural);

port(reset, clk, start: in std_logic;

aboveth: out std_logic;

seg_val_hex: out INTEGER range 0 to 127);

end Count;

architecture impl of Count is

type States is (idle, counting);

signal state, nextstate: States;

signal c, nextc: natural range 0 to 15;

begin

process (state, start, c)

begin

case state is

when idle =>

if start = '1' then

nextstate <= counting;

nextc <= 1;

else nextstate <= idle;

nextc <= 0;

end if;

when counting =>

if (c >= threshold) then

nextstate <= idle;

nextc <= 0;

else nextstate <= counting;

nextc <= c+1;

end if;

end case;

end process;

process (reset, clk)

begin

if (reset = '0') then state <= idle;

c <= 0;

elsif (clk'event and clk='1') then

state <= nextstate;

c <= nextc;

end if;

end process;

aboveth <= '1' when c >=threshold else '0';

-- display counter value on 7-segment output

process(c)

begin

case c is

when 0 => seg_val_hex <= 2#0000001#;

when 1 => seg_val_hex <= 2#1001111#;

when 2 => seg_val_hex <= 2#0010010#;

when 3 => seg_val_hex <= 2#0000110#;

when 4 => seg_val_hex <= 2#1001100#;

when 5 => seg_val_hex <= 2#0100100#;

when 6 => seg_val_hex <= 2#0100000#;

when 7 => seg_val_hex <= 2#0001111#;

when 8 => seg_val_hex <= 2#0000000#;

when 9 => seg_val_hex <= 2#0000100#;

when 10 => seg_val_hex <= 2#0001000#;

when 11 => seg_val_hex <= 2#1100000#;

when 12 => seg_val_hex <= 2#0110001#;

when 13 => seg_val_hex <= 2#1000010#;

when 14 => seg_val_hex <= 2#0110000#;

when others => seg_val_hex <= 2#0111000#;

end case;

end process;

end impl;



SYSTEM

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.numeric_std.all;

-- system

entity System is

port(reset, clk, athome, findfood, lostfood, closetofood,

success, scantimeup: in std_logic;

food: out std_logic;

LEDR : out std_logic_vector(5 downto 0); -- for 6 red leds

LEDG : out std_logic_vector(2 downto 0); -- for 3 green leds

seg_val_hex0: out INTEGER range 0 to 127; -- to display the state

seg_val_hex1: out INTEGER range 0 to 127; -- to display counter 1

seg_val_hex2: out INTEGER range 0 to 127 -- to display counter 2

);

end System;

architecture sys of System is

component Robot is

port(reset, clk,

findfood, -- location of the food has been identified

closetofood, -- moving towards the food

lostfood, -- location of the food has been lost

scantimeup, -- scan time has expired

success, -- the food has been grabbed, or deposited

athome, -- control is given to exit the arena

aboverestth, abovesearchth : in std_logic;

rest, search, food : out std_logic;

LEDR : out std_logic_vector(5 downto 0);

LEDG : out std_logic_vector(2 downto 0);

seg_val_hex0: out INTEGER range 0 to 127);

end component;

component Count is

generic(threshold: natural);

port(reset, clk, start: in std_logic;

aboveth: out std_logic;

seg_val_hex: out INTEGER range 0 to 127);

end component;

signal rest, aboverest, search, abovesearch, ck_1Hz: std_logic := '0';

attribute chip_pin : string;

attribute chip_pin of reset : signal is "R22";

attribute chip_pin of clk : signal is "L1";

attribute chip_pin of athome : signal is "U12";

attribute chip_pin of findfood : signal is "W12";

attribute chip_pin of lostfood : signal is "V12";

attribute chip_pin of closetofood : signal is "M22";

attribute chip_pin of success : signal is "L21";

attribute chip_pin of scantimeup : signal is "L22";

attribute chip_pin of seg_val_hex0 : signal is "J2,J1,H2,H1,F2,F1,E2";

attribute chip_pin of seg_val_hex1 : signal is "E1,H6,H5,H4,G3,D2,D1";

attribute chip_pin of seg_val_hex2 : signal is "G5,G6,C2,C1,E3,E4,D3";

attribute chip_pin of LEDR : signal is "V19,T18,Y19,U19,R19,R20";

attribute chip_pin of LEDG : signal is "V22,U21,U22";

begin

-- Clock divider from 50 MHz to a 1 Hz pulse (ck_1Hz)

process (clk)

variable cnt : INTEGER RANGE 0 TO 67108863;

constant verrou_t : INTEGER := 50000000;

begin

if (clk'event and clk = '1') then

if (reset = '0') or (cnt = verrou_t) then

cnt := 0;

else

cnt := cnt + 1;

end if;

end if;

if (cnt = verrou_t) then

ck_1Hz <= '1';

else

ck_1Hz <= '0';

end if;

end process;

-- instantiate robot

robot_inst: Robot port map(reset, ck_1Hz, findfood, closetofood, lostfood, scantimeup, success, athome,

aboverest, abovesearch, rest, search, food, LEDR, LEDG, seg_val_hex0);

-- instantiate counter for resting

counter1: Count generic map (4) port map(reset, ck_1Hz, rest, aboverest, seg_val_hex1);

-- instantiate counter for searching

counter2: Count generic map (10) port map(reset, ck_1Hz, search, abovesearch, seg_val_hex2);

end sys;

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.numeric_std.all;

-- Configuration of the system

configuration sys_config of System is

for sys

for robot_inst : Robot

use entity work.Robot (intf)

port map (reset => reset,clk => ck_1Hz, findfood => findfood, closetofood => closetofood, lostfood => lostfood, scantimeup => scantimeup, success => success, athome => athome, aboverestth => aboverest, abovesearchth => abovesearch, rest => rest, search => search, food => food, LEDR => LEDR, LEDG => LEDG, seg_val_hex0 => seg_val_hex0);

end for;

for counter1 : Count

use entity work.Count (impl)

generic map (threshold => 4)

port map (reset => reset, clk => ck_1Hz, start => rest, aboveth => aboverest, seg_val_hex => seg_val_hex1);

end for;

for counter2 : Count

use entity work.Count (impl)

generic map (threshold => 10)

port map (reset  => reset, clk => ck_1Hz, start => search, aboveth => abovesearch, seg_val_hex => seg_val_hex2);

end for;

end for;

end sys_config;

